# Autodesk Civil 3D 2007 LandXML Support

Table of Contents

Autodesk Civil 3D 2007 LandXML Support	1

Summary	3

LandXML Schema Versions Supported	3

General data handling	3

What’s New in Civil 3D 2007	3

Import	3

Export	4

Import Details	4

Export Details	6

## Summary

This document details the specific LandXML import export functionality and changes from previous version of Autodesk Civil 3D.

- Improved interoperability with all Registered LandXML Applications www.LandXML.org/LandXMLApps.htm

- Supports all new LandXML-1-1 data.

- Supports new high definition 3D Road model

- Supports Survey data import/export

LandXML Schema Versions Supported

- LandXML-1.1

- LandXML-1.0

- LandXML-0.88

## General data handling

- All coordinate locations are always treated as Northing, Easting, Elevation (or Y,X,Z).

- All station values are treated as the actual measured distance along the alignment or geometry.

- Point references to pointType derived locations are supported, <CgPoint pntRef=”100”/>, including: CgPoint, Start, Center, End, Monument, P and CrossSectPnt elements.

## What’s New in Civil 3D 2007

### Import

- Much faster performance (up to 10x).

- Supports LandXML-1.1, 1.0 and 0.88 data files.

- Control over importing data that may already exist in the drawing (skip, update or append).

- New option to handle unspecified pipe diameter units.

- New option to import surface source data: breaklines, contours and data points as 3d polylines and cogo points.

- Imports breakline and contour surface source data directly into surface TIN definition.

- Import a surface from breakline, contour or data points source data and no TIN definition.

- Imports nested parcels as union parcels.

- Imports PI (Point of Intersection) based Alignments.

- Alignment spiral import supports Clothoid, Bloss, HalfSineWave and Japanese Cubic types.

- Imports Plan Features as 2D/3D polylines.

- Import survey data directly in drawing: monuments, survey monuments, and reduced observations as cogo points in specially named point groups.

- Supports extended survey data import into Survey feature: equipment details, instrument and target setup details, monuments, survey monuments, observation groups, raw observations, reduced observations.

- Imports curved pipes.

### Export

- Much faster performance.

- Exports corridor model as Roadway with alignments, profiles, reference surfaces and design cross sections.

- Exports union parcels as nested parcel elements.

- Exports additional parcel statistic attributes.

- Exports breakline and contour source data for TIN surfaces.

- Exports volume surfaces with statistics.

- Exports curved pipes.

- Option to export sampled cross sections.

- Alignment spiral export supports Clothoid, Bloss, HalfSineWave and Japanese Cubic types.

- Author/User identification data is stored at the system level, no longer requiring entry for each drawing export.

## Import Details

Supported Versions: LandXML-0.88, 1.0 and 1.1

| LandXML Element Supported | Notes |
| --- | --- |
| Units | Used for unit conversion if required |
| Application |  |
| Author |  |
| CoordinateSystem |  |
| CgPoints | Supports multiple elements. If <CgPoints> is named, a corresponding point group is created and cogo points added. |
| CgPoint | Imported a cogo point, uses name and desc or code attributes. Number is auto-assigned if name is alpha-numeric, but name is maintained. |
| Alignments |  |
| CoordGeom | Line, curve, IrregularLine, chain, and spiral (clothoid, bloss, halfSineWave, and Japanese cubic) |
| AlignPI |  |
| StaEq | Station equations |
| Superelevation | All super elevation transitions |
| DesignSpeeds | All design speed station ranges imported |
| Profiles |  |
| ProfAlign | A design vertical alignment (PVI, Vertical curves: circular, parabolic, and asymmetric parabolic) |
| ProfSurf | Imported as static sampled ground surface |
| Parcels | Creates parcels from data, supports nested parcels |
| CoordGeom | Line, curve, spiral, IrregularLine and chain |
| Monuments | Creates cogo points in a specially named monuments points group |
| Surfaces |  |
| SourceData |  |
| Breaklines | Imported as 3D polylines on layer “<surfacename>_Breaklines” |
| Contours | Imported as 3D polylines on layer “<surfacename>_Contours” |
| DataPnts | Imported as cogo point group named “<surfacename>_DataPoints” on layer “<surfacename>_DataPoints” |
| Definition |  |
| TIN | Option to import faces only, then re-triangulate “quick” or maintain the triangulation in the file (slower). Supports face edge visibility and face neighbor optimization attributes |
| Survey |  |
| ReducedObservations | Creates cogo points in a observations point group |
| SurveyMonuments | Creates cogo points in a monuments point group |
| Survey | Survey element |
| InstrumentDetails |  |
| SurveyHeader | Units, CoordinateSystem |
| ObservationGroup | Setups, raw and reduced observations |
| PlanFeatures |  |
| CoordGeom | Imported as 3D polylines to current layer |
| PipeNetworks | Each <PipeNetwork> is imported as a C3D Pipe Network |
| Pipes | Supports curved pipe |
| Units.diameterUnit |  |
| Structs |  |
| Units.diameterUnit |  |

## Export Details

Supported Versions: LandXML-1.1

| LandXML Element Supported | Notes |
| --- | --- |
| Units | Always exported |
| Application | Always exported |
| Author |  |
| CoordinateSystem | Exports Map Zone name / EPSG name / Open Geospatial Consortium’s Well Known Name |
| CgPoints | Each point group exports as <CgPoints> with matching name and all contained points exported |
| Alignments |  |
| CoordGeom | Line, curve, spiral (clothoid, bloss, halfSineWave, and Japanese cubic) |
| StaEq | Station equations |
| Superelevation | All super elevation transitions |
| DesignSpeeds | All design speed station ranges exported with Corridor model to Roadway element. |
| Profiles |  |
| ProfAlign | Design profiles (PVI, Vertical curves: circular, parabolic, and asymmetric parabolic) |
| ProfSurf | Surface sampled profile data |
| CrossSects |  |
| CrossSectSurf | Sample lines exported as CrossSectSurfs |
| DesignCrossSectSurf | Corridor assemblies exported as DesignCrossSectSurfs |
| Parcels |  |
| CoordGeom | Union parcels exported as nested parcels |
| Surfaces |  |
| SourceData |  |
| Breaklines | All exported |
| Contours | All exported |
| DataPnts | All exported |
| Definition |  |
| TIN | Complete TIN with optional watershed |
| GRID | Not supported |
| Roadways | Corridors exported as <Roadways> with references to <Alignments> with Profiles and Cross Sections (both sampled ground and design sections) and to 1 or more reference surfaces |
| PipeNetworks |  |
| Pipes |  |
| Units.diameterUnit |  |
| Structs |  |
| Units.diameterUnit |  |